# Converting MIUI Android OS to Pixel OS on Redmi Note 8

Author: Karan B. Vaya

## Chapter 1: Introduction

### 1.1 Background

The Redmi Note 8 was originally shipped with MIUI, Xiaomi’s custom Android skin. While MIUI provides a visually rich interface, it often impacts performance and includes advertisements, restricted customization, and heavy system services. As a learner in cyber security and ethical hacking, transitioning to a clean and highly controllable environment like PixelOS supports deeper understanding of Android internals.

### 1.2 Purpose of OS Conversion

The purpose was to explore Android system modification, learn flashing methodologies, and reduce system bloat to improve device speed. It also supports future cybersecurity experiments and custom kernel work.

### 1.3 Limitations of MIUI

- System apps cannot be freely removed

- Excessive background services

- Ads inside pre-installed applications

- Lower performance and slower animations over time

### 1.4 Benefits of PixelOS

- Clean stock Android experience similar to Google Pixel devices

- Better privacy controls with no ads

- Smoother animations due to fewer system processes

- Greater customization flexibility this project involves replacing the default MIUI operating system on the Redmi Note 8 smart phone with PixelOS, a clean and secure stock Android-based ROM inspired by Google Pixel devices. The motivation behind this conversion is based on practical learning, personal device optimization, and foundational steps toward cyber security experimentation.

The Redmi Note 8 originally runs MIUI, which is visually enhanced and feature-rich, but it includes system ads, bloatware, and higher resource consumption. Through this conversion, the goal extends beyond performance upgrades — it demonstrates real hands-on experience with mobile operating systems, recovery methods, and ROM flashing, which are crucial skills for cyber security and ethical hacking profiles.

## Chapter 2: Objective

### 2.1 Primary Objective

To replace MIUI with PixelOS in order to achieve a faster, cleaner, and more privacy-focused Android experience while developing practical knowledge in mobile OS customization.

### 2.2 Secondary Learning Objectives

- Understand bootloader unlocking mechanisms

- Learn recovery installation and flash processes

- Perform system partition management and troubleshooting

- Explore possibilities for cybersecurity testing environments like Kali NetHunter The main objective of this project is to remove MIUI due to performance limitations and unwanted bloatware, and install PixelOS to achieve improved performance, better privacy control, and a near-stock Android UI with enhanced features.

## Chapter 3: Hardware & Software Requirements

### 3.1 Hardware Requirements

- Redmi Note 8 smart phone

- USB Type-C cable

- Computer/Laptop with stable USB connectivity

- Minimum 8GB free storage available on device

### 3.2 Software Requirements

- Bootloader unlocked through Xiaomi official unlock process

- OrangeFox custom recovery installed

- PixelOS ROM (Android 13.0 build 2214)

- Platform Tools (ADB & Fastboot drivers)

- Firmware package matching device variant

- Minimum 70% battery to prevent power loss during flashing

### 3.3 Compatibility Considerations

This project required choosing ROM and firmware specific to Redmi Note 8 Ginkgo to avoid soft brick. Additionally, kernel restrictions prevented full Kali NetHunter deployment. Choosing an actively maintained ROM is always recommended for long-term stability. Hardware:

- Redmi Note 8 (Codename: Ginkgo)

- USB cable

- Computer with ADB and Fastboot installed

Software:

- Bootloader unlocked

- OrangeFox Recovery

- PixelOS ROM (Android 13.0 build 2214)

- Proper firmware package

- Platform tools

- Battery charged above 70%

## Chapter 4: Procedure

### 4.1 Bootloader Unlocking

To begin the OS conversion, the bootloader of the Redmi Note 8 had to be unlocked through the official Xiaomi unlock tool. This required:

- Binding Mi Account to the device

- Enabling developer options and OEM unlocking

- Using Fastboot to execute the unlock request this step legally grants access to modify system partitions and install a custom OS.

### 4.2 Custom Recovery Installation

OrangeFox Recovery was installed using Fastboot mode. This custom recovery enables advanced flashing operations that are not possible with the limited stock recovery.

### 4.3 Flashing PixelOS ROM

- Rebooted into OrangeFox Recovery

- Wiped Dalvik/ART Cache, Cache, System, and Data

- Flashed PixelOS ROM (Android 13.0 build 2214)

- Formatted Data to avoid encryption issues

- Rebooted to system

### 4.4 Troubleshooting Bootloop & Soft Brick

On first boot, the device got stuck in a bootloop due to limited ROM maintenance and firmware mismatch. Resolution steps:

- Returned to recovery

- Clean formatted Data and Cache

- Re-flashed ROM properly this restored the system and prevented a full brick situation.

### 4.5 System Setup & Initial Testing

After successful booting:

- PixelOS UI customization completed

- Updates and drivers checked

- Core performance tested

PixelOS provides:

- No system bloatware

- Smooth performance and stock Android controls

- Improved privacy without unwanted tracking

- Clean and fast stock Android experience

- No bloatware and smoother animations

- Improved privacy controls and regular security updates

- Slight decrease in camera optimization compared to MIUI

## Chapter 5: Features Comparison

### 5.1 UI & Experience

MIUI features a heavily modified interface that can feel cluttered due to additional animations and pre-installed apps. PixelOS offers a clean, minimal UI similar to Google Pixel devices, resulting in a more intuitive and fluid user experience.

### 5.2 Performance & Battery

PixelOS improves performance by eliminating unnecessary system processes. Multitasking becomes faster and thermal throttling decreases. Battery backup remains stable with slight improvements in standby time.

### 5.3 Security & Privacy

PixelOS removes ads and tracking services often found in MIUI apps. It benefits from Google's security updates and offers improved user data control.

### 5.4 Update & Support Stability

MIUI has official long-term support but becomes heavier with each update. PixelOS support is community dependent, meaning updates may be less frequent but more optimized.

## Chapter 6: Testing & Results

### 6.1 Performance Tests

After installation, performance was evaluated based on app launch speed, RAM usage, and UI responsiveness. PixelOS showed reduced latency and faster boot time compared to MIUI due to fewer background apps.

### 6.2 Battery Analysis

Battery life remained almost identical to MIUI but standby drain improved. Reduced system ads and tracking contributed to consistent performance.

### 6.3 Stability Observations

During testing, minor glitches occurred because this ROM version was not officially maintained. A soft-brick was encountered but resolved through re-flashing.

### 6.4 Kali NetHunter Compatibility Review

Attempts were made to install Kali NetHunter for ethical hacking practice. However, the required kernel for Redmi Note 8 (Ginkgo) is not officially supported. This limited penetration testing capabilities.

## Chapter 7: Skills Developed

### 7.1 Technical Skills

- Bootloader unlocking and permission handling

- Installation and configuration of custom recovery tools

- ROM flashing and kernel compatibility evaluation

- Handling soft-brick, bootloops, and system recovery

### 7.2 Cybersecurity Relevance

This project enhanced practical understanding of Android internal workings and system-level modifications. These skills form a strong foundation for future work in cybersecurity domains like penetration testing, mobile security auditing, and OS vulnerability research.

## Chapter 8: Conclusion

### 8.1 Key Outcomes

MIUI was successfully replaced with PixelOS, leading to a cleaner UI, improved performance, better privacy control, and deeper system-level learning. The troubleshooting process demonstrated strong resilience and critical problem-solving abilities.

### 8.2 Future Scope & Improvements

To achieve full cybersecurity experimentation capability, a compatible kernel must be found or built for Kali NetHunter support. Additionally, using officially maintained ROM versions will provide smoother update cycles and better long-term stability.

## Chapter 9: References

### 9.1 Source Links & Documentation

- PixelOS Community Resources

- XDA Developers — Redmi Note 8 Custom ROM Guides

- Xiaomi Bootloader Unlock Official Tool

- OrangeFox Recovery Documentation

- Kali Net Hunter Supported Devices List

# Appendix – Cybersecurity Career Profile

# Personal Profile

I am currently in the final year of my Diploma in Computer Science. I am strongly interested in cybersecurity and system security enhancement. Through self-driven practical experiments such as custom ROM flashing, system boot repair, and OS optimization, I am building foundational technical skills required for ethical hacking.

# Motivation for Cybersecurity Studies

Technology today is deeply integrated into every aspect of life, making security critical. I aim to contribute to this field by learning how systems operate at internal layers, identifying vulnerabilities, and strengthening security infrastructures. My OS conversion project is one of the early steps toward this career path—hands-on learning over theoretical assumptions.

# Core Competencies Summary

- Android System Customization & Recovery Operations

- Bootloader, Firmware & Kernel Interaction Knowledge

- Strong troubleshooting mind-set

- Persistent learning approach and technical adaptability

# Future Goals in Cybersecurity

I intend to pursue advanced education in cybersecurity with specialization options such as:

- Mobile Security & Pen-Testing

- Security Operations & Threat Response

- Digital Forensics & Linux-based Exploit Testing

I aim to build expertise in securing devices, systems, and networks that are increasingly targeted by cyber threats. This project stands as a reflection of my commitment to hands-on security learning and technical self-improvement.